MIDDLE EAST TECHNICAL UNIVERSITY

DEPARTMENT OF ELECTRICAL & ELECTRONICS ENGINEERING

TROY TECH

FINAL REPORT

Section: 7

Studio Coordinator: Mustafa Mert ANKARALI

Partners:

Hasan ÖZKARA - 2031888

Kağan ÖZASLAN - 2031854

Caner POTUR - 2031250

Mustafa Ercan OKATAN - 2031193

Mert KAYIŞ - 2030997

Table of Contents

1. Introduction 1

2. Executive Summary 4

3. Design Description 1

3.1 Top-down system descriptions2

3.2 Technical Specifications of Video Transfer Subsystem 2

3.3 Technical Specifications of Command Transmission Subsystem 2

3.4 Technical Specifications of Motor Drive Subsystem 2

3.4.1 Motors 2

3.4.2 Holonomic Control 2

3.4.3 DC Motor Drive Movements 2

3.5 Technical Specifications of Shooting Subsystem: 2

3.6 Technical Specifications of Power Supply Subsystem 2

4. Result and Analyses of Performance Tests4

4.1 Range performance5

4.2 Speed/stability of robot movements5

4.3 Shooting performance5

4.4 Power consumption5

5. List of Deliverables 1

6. Budget 4

6.1 Actual Expenditures 5

6.2 Total Cost 5

7. Conclusion 1

APPENDICES4

APPENDIX A5

APPENDIX B5

3.4 Technical Specifications of Motor Drive Subsystem

3.4.1 Motors

We chose “Namiki 22CL-3501PG” for our motor drive subsystem as shown in Fig.10. Its specifications are given in Table 1.

Figure x. Namiki 22CL-3501PG

Table 1. specifications of Namiki 22CL-3501PG

| Brand | Namiki Coreless Motor (Japan) |
| --- | --- |
| Type | Gear Motor |
| Voltage | 12V |
| Rated power | 15W |
| Stall Torque | 1.6Nm (16Kg · cm) |
| Continuous torque | 0.5Nm |
| Output speed | 120 r / min @ 12VDC |
| Diameter | 22 mm |
| Length | 65 mm |
| Weight | 140 g |

3.4.2 Holonomic Control

We chose holonomic control since its movement is better than the other options. Especially when defending, it is necessary to move left and right parallel to the goal line in order to be able to hold the ball. However, in differential drive method, the robot must turn in that direction in order to go to the right or left. Therefore, he can't see the ball while defending. However, in holonomic control method, the robot can easily go in all directions and defend his goal. The difference between two methods is shown in Fig. x.

Figure x. The difference between holonomic control and differential drive

As shown in Fig. x the robot can move multiple directions (i.e. moving forward, backward, left, right and spinning around it).  It is based on three separately driven wheels placed on the robot body. The direction of the robot can be changed by varying the relative rate of rotation of its wheels. Therefore, it does not require an additional steering motion. Some example motions with varying speed of wheels are shown in Fig. x.

Figure x. Holonomic drive

3.4.3 DC Motor Drive Movements

i) Move Forward

When user moves up the left analog of joystick, robot moves forward. For this operation, the two front DC motors should turn same speed and same rotation. This can be achieved with Arduino by setting the same PWM value to the both front motor pins. Note that different speeds can be obtained by changing the left analog position.

ii) Move Backward

When user moves down the left analog of joystick, robot moves backward. For this operation, the two front DC motors should turn same speed and same rotation. This can be achieved with Arduino by setting the same PWM value to the both front motor pins. Note that different speeds can be obtained by changing the left analog position.

iii) Spin Right

When user moves right the right analog of joystick, robot spins right. For this operation, all three DC motors should turn at the same rotation and same speed. Note that different speeds can be obtained by changing the right analog position.  This can be achieved by changing the PWM values for the motor pins.

iv. Spin Left

When user moves left the right analog of joystick, robot spins left. For this operation, all three DC motors should turn at the same rotation and same speed. Note that different speeds can be obtained by changing the right analog position.  This can be achieved by changing the PWM values for the motor pins.

iv. Move Right

When user moves right the left analog of joystick, robot move right. For this operation, both front DC motors should rotate CCW direction and bottom DC motor should turn CW direction.

iv.  Move Left

When user moves left the left analog of joystick, robot move left. For this operation, both front DC motors should rotate CW direction and bottom DC motor should turn CCW direction.

-

Appendix A-User Manual

USER MANUAL

HELEN-V

May 10, 2019

Table of Contents

1. What’s inside the box 1

2. Quick Start Guide 2

3. Controls 3

4. Charging 4

5. Care and Maintenance 5

- What’s inside the box

Your shipping box contains Helen-V (Fig. 1), Eachine ROTG02 FPV Receiver (Fig. 2), a command transmitter module with PS1 joystick (Fig. 3), Artengo beach volley ball (Fig. 4) and charging cables. Remove all items carefully and do not discard your shipping box.

Figure 1. HELEN-V

Figure 2. Eachine ROTG02 FPV Receiver

Figure 4 . Artengo beach volley ball

Figure 3. Command Transmitter Module

- Quick Start Guide

1

2

5

3

4

5.8 GHz

- Install any FPV receiver app to your smartphone /tablet/PC (“GoFPV” is recommended.).

- Connect “ROTG02 FPV Receiver” to your smartphone /tablet/PC..

- Turn on HELEN-V and command transmission unit

- Adjust the channel of “ROTG02 FPV Receiver” by pressing the adjust buttons. If you press any of these buttons 2 seconds, it will automatically set the channel.

- You can start playing the game!

- Controls

- Charging

Helen-V includes 2 power-banks and a 12V (2.8Ah) accumulator battery. Moreover, command transmission system has a power bank. You can charge the power-banks with a simple micro-usb cable. You can charge 12V battery using DC supply or a 12 V battery charging adaptor.

WARNING:

- While charging the 12 V accumulator battery, make sure that other power-banks are 		 switched off.

- Reverse charging is prohibited! Do not reverse the positive (+) and negative (-) terminals when charging the 12V accumulator battery.

- Care and Maintenance

Recharge the batteries after each use. Only an adult can handle the battery. Recharge the battery at least once a month when the robot is not being used.

Store the robot where the temperature is between -20º and 45º C (-4º to 113º F). Don’t leave it in your car, because temperatures in parked cars can exceed this range.

It does not have a waterproof function, so keep the robot out of water.

Do not use the robot in loose dirt, sand or fine gravel which could damage moving parts, motors or the electric system.

When not using, all the electrical source should be turn off.

In case of a technical problem, please contact Troy Tech, please do not try to fix the problem by yourself

Appendix B