MIDDLE EAST TECHNICAL UNIVERSITY

DEPARTMENT OF ELECTRICAL & ELECTRONICS ENGINEERING

TROY TECH

WEEKLY REPORT #10

12.12.2018-20.12.2018

Section : 7

Studio Coordinator: Mustafa Mert ANKARALI

Partners :

Hasan ÖZKARA - 2031888

Kağan ÖZASLAN - 2031854

Caner POTUR - 2031250

Mustafa Ercan OKATAN - 2031193

Mert KAYIŞ – 2030997

Progresses for Previous Week:

- We tested the command transfer unit using NRF24L01 with 15dBi 2.4Ghz linear antenna and 25dbi  2.4GHz Yagi antenna alternately. Both antennas increased the maximum transmission range widely.

- We bought a new FPV transmitter: Eachine TS5828S 40CH 5.8G 600MW. It can transmit the video from the 1st floor of E building to ground, first and second floor of D building and up to study hall in D building smoothly.

- We created a RC robot using two DC motors, power units, FPV transmitter and command receiver for demo:

- We created a transmitter unit including PS1 joystick and transmitter. By using this unit, robot can move forward-backward and turn around itself with different speeds. Speed can be adjusted using analogs(3 different speed options for all movements.)

Plans for the Next Week:

- Writing conceptional design report

- Preparing a presentation

- Testing the robot